SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO4: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 30%

Annexure A

Industry Problem Solving Task

Code of Conduct:

I  _B.Vinayani 192472179 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

## 1. Network Design for a 20-Person Startup

### Recommended Topology: Star Topology

A star topology is the most suitable choice for a small startup because every device is directly connected to a central network switch. The central switch acts as the main communication point between computers, printers, IP phones, and other network devices.

### Network Structure

Internet

|

Router/Firewall

|

Central Switch

_________|_________

|     |     |       |

PC-1   PC-2  PC-3   Printer

|     |     |

Employee Devices

|

Wi-Fi AP

### Required Components

- 24-port managed switch

- Router/firewall

- Cat6 Ethernet cables

- Wi-Fi access point

- Desktop/laptop computers

- Network printer

- VoIP phones, if required

- UPS for networking equipment

### Working

Each employee's computer is connected to the central switch using Cat6 cable. The switch forwards data between devices within the local network. The router provides Internet connectivity and can perform DHCP, NAT, and firewall functions. Wireless access points allow laptops and mobile devices to connect without cables.

### Advantages

- Easy to install and manage.

- Easy troubleshooting.

- Failure of one cable affects only one device.

- Easy to add new employees.

- Provides good network performance.

- Centralized security and monitoring.

### Disadvantage

The main disadvantage is that the central switch is a single point of failure. If the switch fails, connected devices lose network connectivity.

### Justification

For a 20-person startup, reliability, simplicity, and scalability are more important than complicated redundancy. Therefore, star topology provides a cost-effective and manageable solution.

# 2. Network Design for a Bank

### Recommended Topology: Partial Mesh / Full Mesh

Banks require extremely high availability because network failures can affect ATMs, online banking, financial transactions, and internal banking systems. Therefore, a mesh topology is appropriate.

### Network Structure

Internet 1

|

Router 1

/      \

/        \

Core SW1 ----- Core SW2

|\           /|

| \         / |

|  \       /  |

Server ----- Server

\

Router 2

|

Internet 2

### Required Components

- Redundant routers

- Core Layer 3 switches

- Firewalls

- Fiber optic connections

- Servers

- UPS and backup power

- RAID storage

- Backup Internet connections

### Working

In a mesh network, multiple devices have multiple communication paths. If one link fails, routing protocols such as OSPF or BGP can select an alternative path.

For example:

Normal:

Server → Switch 1 → Router 1 → Internet

If Router 1 fails:

Server → Switch 2 → Router 2 → Internet

The network can therefore continue operating even when a component fails.

### Advantages

- Very high reliability.

- Fault tolerance.

- Multiple alternative paths.

- Reduced downtime.

- Suitable for mission-critical applications.

- Supports redundant Internet connections.

### Disadvantages

- High installation cost.

- Complex configuration.

- Requires skilled network administrators.

- More cables and networking equipment.

### Justification

Although mesh topology is expensive, the cost is justified in banking because network availability and reliability are critical. A partial mesh is generally more practical than a complete full mesh because it provides redundancy where it matters most while controlling cost and complexity.

# 3. Network Design for a Small Classroom Lab

### Recommended Topology: Bus Topology

A bus topology connects multiple devices to a common communication backbone. It was historically attractive for small networks because it required less hardware.

### Basic Structure

Terminator                              Terminator

|                                        |

|------------ Backbone Cable ------------|

|          |          |

PC1        PC2        PC3

Traditional Ethernet bus networks used coaxial cable, T-connectors, and terminators.

### Components

- Coaxial backbone cable

- T-connectors

- BNC connectors

- 50-ohm terminators

- Network interface cards

### Working

When a computer sends data, the signal travels along the shared backbone. All connected computers can detect the signal, but only the intended destination processes the data.

### Advantages

- Low initial cost.

- Requires less cable.

- Simple structure.

- Suitable for small networks.

- Does not require a central switch in the traditional implementation.

### Disadvantages

- Backbone failure can affect the entire network.

- Network performance decreases as devices increase.

- Collisions can occur.

- Troubleshooting can be difficult.

- Limited scalability.

### Justification

For a very small classroom with limited traffic and a strict budget, the low cost of a bus network can be attractive. However, modern Ethernet classroom networks normally use a star topology with an inexpensive switch, because traditional bus Ethernet is obsolete and no longer common in new installations.

# 4. Network Design for a Large University Campus

### Recommended Topology: Hybrid Topology

A university has many buildings and thousands of users. Therefore, one topology alone is not sufficient. A hybrid topology combines different topologies according to the requirements of each part of the campus.

### Network Structure

Internet

|

Core Router

|

=================

Campus Core

Ring / Mesh

=================

/       |       \

/        |        \

Engineering   Library   Hostel

Building     Building   Block

|            |         |

Distribution Distribution Distribution

Switch       Switch      Switch

|            |         |

Access Switch Access SW  Access SW

/ | \         / | \       / | \

PCs PCs APs    PCs APs     PCs APs

### Network Layers

#### Core Layer

Provides high-speed communication between buildings.

#### Distribution Layer

Connects individual buildings and provides routing, policies, and redundancy.

#### Access Layer

Connects end-user devices such as computers, printers, cameras, and wireless access points.

### Components

- Core Layer 3 switches

- Distribution switches

- Access switches

- Fiber optic backbone

- Cat6 Ethernet cables

- Wireless access points

- Routers and firewalls

- Servers

- Network monitoring systems

### Working

High-speed fiber connects different buildings to the campus core. Each building has distribution switches, which connect to access switches. Access switches connect individual computers and other devices.

Wireless access points can be connected using Power over Ethernet (PoE), allowing both data and electrical power to travel through the same Ethernet cable.

### Advantages

- Highly scalable.

- Good performance.

- Fault tolerance.

- Easy management.

- Supports thousands of users.

- New buildings can be added easily.

- Different technologies can be used according to requirements.

### Disadvantages

- Higher installation cost.

- Requires proper network planning.

- More complicated than a simple star network.

- Requires professional network administration.

### Justification

A hybrid design is ideal because different sections of a university have different requirements. The backbone requires redundancy and high speed, while individual buildings require simple and manageable connections.

# 5. Network Design for a Company with Multiple Departments

### Recommended Design: Hierarchical Topology with VLANs

A company with departments such as HR, Finance, Sales, and IT needs network separation for security and efficient management.

### Example Network

Internet

|

Router/Firewall

|

Layer 3 Core Switch

_________|_________

|         |         |

VLAN 10    VLAN 20    VLAN 30

|         |         |

HR      Finance     Sales

|         |         |

Access SW  Access SW  Access SW

/|\        /|\        /|\

PCs        PCs        PCs

|

VLAN 40

|

IT

### Example VLAN Configuration

| Department | VLAN | Example Network |
| --- | --- | --- |
| HR | VLAN 10 | 192.168.10.0/24 |
| Finance | VLAN 20 | 192.168.20.0/24 |
| Sales | VLAN 30 | 192.168.30.0/24 |
| IT | VLAN 40 | 192.168.40.0/24 |

### Working

Each department is placed into a separate VLAN (Virtual Local Area Network). Devices in the same VLAN can communicate normally, while communication between different VLANs is controlled through a Layer 3 switch or router.

For example:

HR PC → HR VLAN

↓

Layer 3 Switch

↓

Finance VLAN

↓

Finance PC

The Layer 3 switch can apply rules determining which departments are allowed to communicate.

### Security Example

HR may need access to employee records but should not necessarily have access to Finance's financial systems.

HR ───────→ HR Servers       ✓

HR ───────→ Finance Servers  ✗

IT ───────→ All Departments  ✓

### Advantages

- Improved security.

- Reduced broadcast traffic.

- Better network organization.

- Easier troubleshooting.

- Easy department expansion.

- Better control over inter-department communication.

### Disadvantages

- VLAN configuration is more complex.

- Requires managed switches.

- Inter-VLAN routing requires Layer 3 functionality.

- Poor configuration can cause security problems.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Problem Analysis | 15% | Clearly identifies and deeply analyzes the industry problem with all factors | Good understanding with minor gaps | Basic understanding; some key aspects missing | Poor or incorrect understanding of the problem |
| Solution Design | 20% | Innovative, practical, and industry-relevant solution | Effective solution with minor limitations | Partially suitable solution | Inappropriate or unclear solution |
| Technical Implementation | 20% | Fully functional, accurate, and well-executed implementation | Mostly functional with minor errors | Partially implemented solution | Incomplete or non-functional implementation |
| Use of Tools & Technologies | 10% | Appropriate and advanced use of relevant tools and technologies | Correct use of tools | Limited or basic use of tools | Incorrect or no use of tools |
| Problem-Solving Approach | 10% | Logical, systematic, and well-structured approach | Mostly logical approach | Somewhat disorganized approach | No clear approach |
| Innovation & Creativity | 10% | Highly creative and original solution | Some creativity | Limited originality | No creativity |
| Testing & Validation | 5% | Thorough testing with real-world scenarios | Adequate testing | Minimal testing | No testing |
| Documentation & Presentation | 5% | Clear, professional, and well-documented | Good documentation | Basic documentation | Poor or no documentation |
| Teamwork / Collaboration | 5% | Excellent coordination and contribution (if team-based) | Good collaboration | Limited participation | No collaboration or contribution |