SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE CODE:  CSA0713                    COURSE NAME: COMPUTER NETWORKS

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

Code of Conduct:

I _B.Vinayani 192472179_ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

## 1. CRC (Cyclic Redundancy Check)

Given:

- Message = 11100011

- Generator polynomial = X3+1X^3 + 1X3+1

Polynomial in binary:

- X3+1=1001X^3 + 1 = 1001X3+1=1001

- Degree = 3

### (i) Find the transmitted message

#### Step 1: Append 3 zeros

Original message = 11100011

After appending zeros = 11100011000

#### Step 2: Perform Modulo-2 division by 1001

Dividend = 11100011000

Divisor  = 1001

Performing XOR division gives:

Remainder = 111

#### Step 3: Add the remainder

Original message = 11100011

CRC bits         = 111

Transmitted message = 11100011111

### Answer (i)

- CRC bits = 111

- Transmitted message = 11100011111

### (ii) Leftmost bit is inverted

Original transmitted data:

11100011111

Leftmost bit changes from 1 to 0:

01100011111

The receiver divides:

01100011111 ÷ 1001

The remainder obtained is:

110

Since:

Remainder ≠ 000

the CRC check fails.

### How does the receiver detect the error?

The receiver always performs CRC division.

- If remainder = 000 → No error detected.

- If remainder ≠ 000 → Error detected.

### Answer (ii)

- Received data = 01100011111

- CRC remainder = 110

- Error is detected because the remainder is non-zero.

## 2(a). Throughput Calculation

Given:

- Bandwidth = 10 Mbps

- Frames per minute = 12,000

- Bits per frame = 10,000

### Step 1: Total bits per minute

= 12,000 × 10,000

= 120,000,000 bits/minute

### Step 2: Convert into bits per second

120,000,000 / 60

= 2,000,000 bits/s

### Throughput

= 2 Mbps

### Answer

- Throughput = 2 Mbps

## 2(b). Channel Capacity

Given:

- Bandwidth (B) = 3000 Hz

- SNR = 3162

Using Shannon's theorem:

C = B log₂(1 + SNR)

Substituting:

C = 3000 log₂(3163)

Since:

log₂(3163) ≈ 11.63

Therefore:

C = 3000 × 11.63

= 34,890 bps

### Answer

- Channel capacity ≈ 34.9 kbps

## 3. Hamming (12,8) Error Correcting Code

### Scenario

Wireless patient monitoring devices continuously transmit:

- Heart rate

- Blood pressure

Wireless communication may suffer from:

- Electromagnetic interference

- Single-bit errors

The hospital uses:

Hamming (12,8)

Meaning:

- 8 data bits

- 4 parity bits

- Total = 12 bits

### Working

Sender:

8-bit data

Adds 4 parity bits

12-bit Hamming code transmitted

If one bit changes during transmission:

101011001101

101111001101

Receiver performs:

- Parity checking.

- Calculates syndrome bits.

- Identifies the exact erroneous bit.

- Corrects the bit immediately.

### Advantages

- Detects single-bit errors.

- Corrects single-bit errors.

- No retransmission required.

- Suitable for real-time medical applications.

### Result

Doctors receive:

- Accurate patient data.

- Faster response.

- Reliable wireless monitoring.

## 4(a). IPv4 Subnetting

Given:

IP address block = 172.16.10.0/24

Subnet mask = /27

### Step 1: Number of subnets

Borrowed bits:

27 - 24 = 3 bits

Number of subnets:

2^3 = 8

### Step 2: Hosts per subnet

Remaining host bits:

32 - 27 = 5

Total addresses:

2^5 = 32

Usable hosts:

32 - 2 = 30

### Answer

- Total subnets = 8

- Usable hosts = 30

### Step 3: Subnet ranges

Block size:

256 - 224 = 32

Subnets:

| Subnet | Range |
| --- | --- |
| 0 | 0-31 |
| 1 | 32-63 |
| 2 | 64-95 |
| 3 | 96-127 |
| 4 | 128-159 |
| 5 | 160-191 |
| 6 | 192-223 |
| 7 | 224-255 |

### IP = 172.16.10.78

78 lies between:

64-95

Hence:

Subnet = 172.16.10.64/27

Network address:

172.16.10.64

Broadcast address:

172.16.10.95

Host range:

65 to 94

### IP = 172.16.10.95

This is:

Broadcast address

Therefore:

- It belongs to the same subnet.

- It cannot be assigned to a host.

### Answer

| Item | Answer |
| --- | --- |
| Subnets | 8 |
| Hosts/subnet | 30 |
| Subnet | 172.16.10.64/27 |
| Network address | 172.16.10.64 |
| Broadcast address | 172.16.10.95 |
| Same subnet? | Yes |
| Usable host? | No |

## 4(b). IPv6

Given:

2001:0db8:0000:0000:0000:ff00:0042:8329

### Compressed form

Remove leading zeros:

2001:db8:0:0:0:ff00:42:8329

Compress consecutive zeros:

2001:db8::ff00:42:8329

### Expanded form

2001:0db8:0000:0000:0000:ff00:0042:8329

### Prefix length

/64

The first four groups are the network portion:

2001:0db8:0000:0000

Hence:

Network prefix:

2001:db8:0:0::/64

### Number of host addresses

Host bits:

128 - 64 = 64

Total:

2^64

Value:

18,446,744,073,709,551,616

### Answer

| Item | Answer |
| --- | --- |
| Compressed | 2001:db8::ff00:42:8329 |
| Expanded | 2001:0db8:0000:0000:0000:ff00:0042:8329 |
| Network Prefix | 2001:db8:0:0::/64 |
| Host addresses | 2^64 |

## 5. University Online Examination Scenario

### Primary Networking Problem

The main networking problem is:

Flow Control

The server transmits data much faster than many receivers can process.

### Effects of Speed Mismatch

#### Sender

10 Gbps

#### Receivers

- 100 Mbps

- 20 Mbps

- 5 Mbps

- 10 Mbps

The sender transmits packets rapidly, while slower receivers cannot:

- Receive packets.

- Process packets.

- Store packets

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |