SIMATS ENGINEERING

Co3-ASSESSMENT TOOL 2

Case study

COURSE NAME: Computer Networks			COURSE CODE:	CSA07

COURSE OUTCOME COVERED

CO3: Analyze the performance of core network protocols such as TCP, UDP, HTTP, and DNS under varying conditions

Assessment Tool Weightage: 40%

Dr.V.Parthipan

QUESTIONS:4							           Total Marks:40

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

| Criteria | Understanding the case 2.5 | Application of Concepts 2.5 | Analysis 2 | Solution Design 2 | Clarity 1 | Total (10) |
| --- | --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |  |
| Q2 |  |  |  |  |  |  |
| Q3 |  |  |  |  |  |  |
| Q4 |  |  |  |  |  |  |

Faculty Signature

# 1. Real-Time Video Conferencing – TCP/UDP and Application Buffering

### Scenario Analysis

The video conferencing system experiences:

- 8% packet loss

- 180 ms jitter

- Fluctuating bandwidth

- Voice distortion

- Frozen video

- Delayed communication

These are serious problems for a real-time multimedia application.

### 1. Is the problem caused by TCP/UDP or the Application Layer?

The problem is primarily related to the transport protocol and application-level buffering strategy, rather than only one layer.

Real-time video conferencing generally prefers UDP-based transport because packets must arrive quickly. TCP provides reliable delivery using acknowledgments and retransmissions, but retransmission of delayed packets can increase latency.

With 8% packet loss and 180 ms jitter, UDP can deliver packets quickly but may result in missing audio/video frames. If the application uses a large playback buffer, it may compensate for jitter but introduce additional delay.

Therefore, both network/transport behavior and application buffering need to be considered.

### 2. Why UDP is Preferred for Real-Time Multimedia

Real-time applications prioritize low latency over perfect reliability.

For example, if a video packet containing an old frame is lost:

Frame 1 → Received

Frame 2 → Lost

Frame 3 → Received

Retransmitting Frame 2 after a long delay may be useless because the application has already moved to Frame 3.

TCP attempts:

Lost Packet

↓

Retransmission

↓

Waiting for packet

↓

Additional delay

This can cause head-of-line blocking, where later data waits for earlier missing data.

UDP avoids this retransmission behavior and allows newer packets to continue arriving.

### 3. Recommended Improvements

#### Transport-Level Improvements

- Use UDP-based real-time transport, such as RTP/RTCP.

- Apply congestion control suitable for real-time traffic.

- Prioritize voice/video traffic using QoS.

- Use packet-loss concealment and forward error correction where appropriate.

- Use adaptive bitrate techniques.

#### Application-Level Improvements

- Use an adaptive jitter buffer instead of a fixed large buffer.

- Dynamically reduce video resolution when bandwidth decreases.

- Reduce frame rate during congestion.

- Prioritize audio over video because voice requires lower delay.

- Drop outdated video frames instead of retransmitting them.

- Monitor packet loss, jitter, latency, and bandwidth continuously.

### Conclusion

The main issue is the combination of high packet loss, high jitter, and inappropriate buffering/transport behavior. A UDP-based real-time design with adaptive buffering, congestion control, QoS, and adaptive video quality can significantly improve conferencing quality.

# 2. Global DNS Architecture

### Scenario Analysis

The company has an average DNS lookup time of 800 ms, which is extremely high for a web application. DNS resolution should normally add only a small portion of the overall page-loading time.

The target is to reduce DNS latency to below 50 ms.

## 1. Reasons for High DNS Resolution Time

Possible causes include:

### a) Centralized DNS Server

If users from different continents contact one DNS server, packets must travel long distances.

User Asia ───────────────┐

User Europe ─────────────┤

User America ────────────┤ → Single DNS Server

This increases network round-trip time.

### b) Network Congestion

During peak periods, congestion can increase DNS request and response times.

### c) Lack of DNS Caching

If DNS records are not cached, clients repeatedly perform complete DNS lookups.

### d) Poor DNS Server Capacity

An overloaded DNS server can take longer to process queries.

### e) Improper TTL Configuration

Very short TTL values cause cached records to expire frequently, increasing the number of DNS queries.

# 2. Proposed DNS Architecture

A distributed Anycast DNS architecture should be used.

Global Users

|

Anycast DNS Address

|

┌──────────────┼──────────────┐

↓              ↓              ↓

DNS Asia        DNS Europe      DNS America

|              |              |

└──────────────┼──────────────┘

↓

Authoritative DNS

### Anycast DNS

Multiple DNS servers use the same IP address.

The routing system directs users to a nearby/optimal DNS server.

For example:

Chennai User → Asia DNS

London User  → Europe DNS

New York User → America DNS

This reduces network round-trip time.

### DNS Caching

DNS responses should be cached at:

- Client devices

- Operating systems

- Browsers

- Local DNS resolvers

- ISP resolvers

Cached responses eliminate repeated lookups.

### DNS Pre-fetching

Frequently accessed domain information can be resolved before it is actually required.

For example:

Homepage

↓

Pre-fetch DNS

↓

API / Image / CDN domain

↓

Faster subsequent request

### TTL Optimization

TTL determines how long a DNS record can remain cached.

A suitable TTL should be selected based on how frequently the record changes.

Too short:

Low TTL → Frequent queries → Higher DNS traffic

Too long:

High TTL → Longer caching → Slower propagation of changes

A balanced TTL can reduce DNS queries while maintaining reasonable update flexibility.

## 3. Evaluation

| Feature | Proposed Architecture |
| --- | --- |
| Performance | Very high |
| Latency | Reduced |
| Scalability | High |
| Availability | High |
| Fault tolerance | High |
| Global coverage | Excellent |

### Advantages

- Lower DNS latency.

- Handles millions of users.

- Provides geographical distribution.

- Failure of one DNS location does not necessarily stop service.

- Reduces load on individual DNS servers.

- Improves user experience.

### Limitations

- Higher infrastructure cost.

- More complex configuration.

- Anycast routing requires careful network management.

- DNS failures can still occur due to configuration errors.

- TTL changes may take time to propagate.

### Conclusion

A combination of Anycast DNS + DNS caching + pre-fetching + optimized TTL values can significantly reduce DNS lookup latency and improve availability. With geographically distributed DNS nodes, achieving a target below 50 ms is realistic for many users, although the exact latency depends on network conditions and user location.

# 3. Distributed DNS for Global Cloud Services

## 1. Impact of Centralized DNS Architecture

The company currently sends all DNS queries to one primary DNS server.

Millions of Users

|

↓

Single DNS Server

|

↓

Overload

This creates a single point of failure.

During promotional events, the number of queries increases sharply. The server becomes overloaded, resulting in:

- Increased DNS response time.

- Higher CPU and memory usage.

- Longer application startup time.

- Increased timeout probability.

- Poor user experience.

- Complete DNS failure if the server becomes unavailable.

Since the reported DNS latency is 800 ms, users experience a significant delay before connecting to the actual cloud service.

# 2. Distributed DNS Solution

A globally distributed DNS architecture should be implemented.

Global Users

|

DNS Query

|

┌─────────┴─────────┐

↓                   ↓

Anycast Network      DNS Caching

|

┌───────────┼───────────┐

↓           ↓           ↓

Asia        Europe      America

DNS          DNS          DNS

|           |            |

└───────────┼────────────┘

↓

Authoritative DNS

### Main Components

1. Anycast DNS:
 Routes users to the nearest or best-performing DNS server.

2. Multiple DNS Servers:
 Servers are deployed across different geographic regions.

3. DNS Caching:
 Frequently requested records are stored by recursive resolvers.

4. Secondary/Backup DNS:
 Provides service if the primary authoritative infrastructure fails.

5. Load Distribution:
 Queries are distributed instead of being sent to a single server.

# 3. TTL, Anycast and DNS Caching

### TTL

TTL specifies how long DNS information can be cached.

High TTL:

- Fewer DNS queries.

- Lower server load.

- Better performance.

- But changes take longer to propagate.

Low TTL:

- Faster DNS record changes.

- More frequent queries.

- Higher DNS traffic.

Therefore, an appropriate TTL should be selected according to the application's requirements.

### Anycast Routing

Anycast allows multiple DNS servers to advertise the same IP address.

User → Same DNS IP

↓

Routing selects

nearest DNS node

Benefits:

- Lower latency.

- Load distribution.

- Better global performance.

- Improved availability.

- Automatic routing around failed locations.

### DNS Caching

Caching stores DNS responses temporarily.

For example:

First Request

User → DNS → Authoritative Server

Later Requests

User → Cached DNS Response

The second type of request is much faster because the complete DNS lookup does not have to be performed again.

## Evaluation

| Factor | Centralized DNS | Distributed DNS |
| --- | --- | --- |
| Latency | High | Low |
| Scalability | Poor | High |
| Availability | Low | High |
| Fault tolerance | Poor | High |
| Server load | Very high | Distributed |
| Global users | Poor performance | Better performance |

### Conclusion

A distributed DNS architecture using Anycast routing, caching, optimized TTL values, and multiple authoritative servers eliminates the single point of failure and significantly reduces DNS latency.

# 4. TCP Latency in High-Frequency Stock Trading

### Scenario Analysis

The financial institution processes thousands of orders per second. During market opening:

Normal acknowledgment latency: 1 ms
 Peak latency: 40 ms

The increase is associated with:

- TCP retransmissions

- Longer connection establishment

- Excessive buffering

Three important TCP-related factors are flow control, Nagle's algorithm, and SYN queue limitations.

## 1. Flow Control

TCP uses a receive window to prevent the sender from transmitting more data than the receiver can process.

Sender →→→→→ Receiver

Window

If the receiver's buffer becomes full, it advertises a smaller receive window.

Eventually:

Receive Window ↓

↓

Sender slows down

↓

Order processing delayed

### Effect on Trading

In high-frequency trading, thousands of small messages may arrive rapidly. A small receive window or overloaded receiver can restrict throughput and increase acknowledgment latency.

### Solution

- Optimize TCP receive/send buffer sizes.

- Monitor receive-window utilization.

- Ensure sufficient system memory.

- Tune network stack parameters.

# 2. Nagle's Algorithm

Nagle's algorithm combines small TCP packets to reduce network overhead.

For normal applications, this can improve efficiency.

However, trading systems frequently send small, latency-sensitive messages.

Example:

Order 1 → Small packet

Order 2 → Small packet

Order 3 → Small packet

Nagle may wait for acknowledgment before sending additional small data.

This can introduce unnecessary delay.

### Effect on Trading

A small delay in sending an order can affect:

- Order execution.

- Trade confirmation.

- Market opportunities.

- Customer experience.

### Solution

For latency-sensitive connections, disable Nagle's algorithm using TCP_NODELAY, when appropriate.

TCP_NODELAY = Enabled

This allows small packets to be transmitted immediately instead of waiting to combine them.

# 3. SYN Queue Limitations

Before TCP communication begins, the three-way handshake occurs:

Client → SYN

Server → SYN-ACK

Client → ACK

During market opening, thousands of new connections may arrive simultaneously.

The server maintains a SYN queue for partially established connections.

If the queue becomes full:

Many SYN requests

↓

SYN Queue Full

↓

Dropped/Delayed SYN requests

↓

Retransmissions

↓

Connection establishment delay

### Effect on Trading

Longer connection establishment directly increases transaction latency.

It can cause:

- Delayed order submissions.

- TCP retransmissions.

- Connection failures.

- Increased server load.

### Solution

- Increase SYN backlog/queue capacity.

- Use SYN cookies where appropriate.

- Optimize connection handling.

- Prefer persistent TCP connections for trading sessions instead of repeatedly creating new connections.

- Monitor connection and SYN queue utilization.

# 2. Effect on High-Frequency Transaction Processing

| Factor | Effect | Result |
| --- | --- | --- |
| Flow Control | Limits data transmission | Increased delay |
| Nagle's Algorithm | Delays small packets | Higher order latency |
| SYN Queue | Delays connection establishment | Connection delays |

The combination can increase acknowledgment latency from 1 ms to 40 ms.

# 3. Recommended TCP Configuration

### Flow Control

- Tune TCP send/receive buffers.

- Prevent receiver buffer exhaustion.

- Monitor TCP window utilization.

### Nagle's Algorithm

Use:

TCP_NODELAY

for latency-sensitive order messages where appropriate.

### SYN Queue

- Increase SYN backlog.

- Enable SYN cookies when appropriate.

- Maintain persistent connections.

- Monitor connection establishment rates.

### Additional Improvements

- Use dedicated low-latency network paths.

- Apply QoS to trading traffic.

- Reduce unnecessary application buffering.

- Optimize server CPU and network processing.

- Monitor TCP retransmissions continuously.

## Final Conclusion

For high-frequency trading, low latency is more important than maximizing bulk throughput. Flow-control limitations can restrict transmission, Nagle's algorithm can delay small messages, and SYN queue limitations can delay new connections. Proper TCP tuning, persistent connections, TCP_NODELAY, adequate buffers, and optimized SYN handling can reduce latency while maintaining reliable order delivery.

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Case | 25 | Thorough understanding; clearly identifies all key issues | Good understanding with most issues identified | Basic understanding; some issues missed | Poor understanding; problem not identified correctly |
| Application of Concepts | 25 | Effectively applies relevant concepts to analyze the case deeply | Applies appropriate concepts with minor gaps | Limited application of concepts | Fails to apply relevant concepts |
| Analysis | 20 | In-depth analysis with strong reasoning and insights | Good analysis with logical reasoning | Basic analysis with limited depth | Weak or no analysis; lacks reasoning |
| Solution Design | 20 | Innovative, feasible solutions with strong justification | Logical solutions with adequate justification | Basic solutions with weak justification | Incorrect or no solution provided |
| Clarity | 10 | Clear, well-structured, and easy to understand | Organized and understandable presentation | Limited clarity and structure | Poor clarity; difficult to understand |